# TAS58xx EQ Control Proposal v2

Simpler EQ control for the tas58xx component. Proposal by @mrtoy-me for review and comment. Replaces v1.

# Summary

eq_mode becomes a switch (On / Off). On at first boot, then the last setting is restored.

One fixed_filters list on the audio_dac (no slider). Any filter type can be used.

Two slider lists on the number platform: gain_bands (the slider sets the gain) and frequency_bands (the slider sets the frequency).

Every entry has a channel (left, right or both) and an always_on option.

Fixed filters and bands can all be combined on a channel, up to 15 biquads per channel.

## Changes from v1

EQ presets are removed.

left_fixed_filters and right_fixed_filters become one fixed_filters list with a channel option. Fixed filters can now be any filter type, not only notch and band pass.

stereo_bands, left_bands and right_bands become gain_bands with a channel option.

New: frequency_bands, a slider that sets the frequency of a filter.

fixed_filters_always_on is replaced by an always_on option on each entry.

PBTL: channel: right is a config error (v1 gave a warning).

# YAML

audio_dac:

- platform: tas58xx

id: dac1

fixed_filters:              # optional, no slider; any filter type

- eq_filter: notch

frequency: 50Hz

q_factor: 10.0

channel: both           # left | right | both, default both

always_on: true         # default true for fixed filters

- eq_filter: peaking

frequency: 120Hz

q_factor: 1.0

gain: -4dB              # only for filters with a gain input

channel: left

always_on: false

switch:

- platform: tas58xx

eq_mode:

name: EQ

restore_mode: RESTORE_DEFAULT_ON  # default

number:

- platform: tas58xx

tas58xx_id: dac1

gain_bands:                 # slider = gain; filters with a gain input only

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

q_factor: 0.707

channel: both

always_on: false        # default false for bands

- name: Mid

eq_filter: peaking

frequency: 1kHz

q_factor: 2.0

channel: both

always_on: true

frequency_bands:            # slider = frequency; any filter type

- name: Crossover

eq_filter: high_pass

gain: 0dB               # only for filters with a gain input

q_factor: 0.707

channel: right

always_on: true

- name: Notch

eq_filter: notch

q_factor: 8.0

channel: left

always_on: false

channel and always_on are written out in this example even where they are the default.

# Entry Options

| Option | fixed_filters | gain_bands | frequency_bands |
| --- | --- | --- | --- |
| eq_filter | required, any filter | optional, default equalizer; filters with a gain input only | optional, default equalizer; any filter |
| frequency | required, 20 Hz to 16 kHz | required, 20 Hz to 16 kHz | the slider; starts at 1000 Hz |
| gain | optional, default 0 dB; filters with a gain input only | the slider; starts at 0 dB | optional, default 0 dB; filters with a gain input only |
| q_factor | optional, default per filter (table below) | optional, default per filter | optional, default per filter |
| channel | optional, default both | optional, default both | optional, default both |
| always_on | optional, default true | optional, default false | optional, default false |

The options allowed depend on eq_filter: for example gain on a notch is a config error.

Each band is a full ESPHome number, so standard options such as id, icon, mode and entity_category can be added.

A band's last value is restored at boot. Before anything is saved, gain sliders start at 0 dB and frequency sliders at 1000 Hz. There is no initial value option for now.

## Filters

| eq_filter | Gain input | Default q_factor | Why |
| --- | --- | --- | --- |
| equalizer (default), peaking | yes | 2.144908 | 2/3-octave constant Q, the ISO 15 band graphic EQ |
| low_shelf, high_shelf | yes | 0.707 | standard shelf (RBJ slope 1) |
| low_pass, high_pass | yes | 0.707 | Butterworth, the industry standard |
| notch, band_pass | no | required | no common default: it depends on what is removed or isolated |

q_factor is the only width input: it is the norm for parametric EQ (DAWs, miniDSP, Equalizer APO, REW, AutoEQ), so values can be copied straight in.

Gain limits are as v1: -24 to +24 dB, +18 dB for high_shelf and high_pass.

## Slider Ranges (number platform)

number:

- platform: tas58xx

gain_min: -15dB             # gain_bands slider range

gain_max: 15dB

frequency_min: 20Hz         # frequency_bands slider range, default 20 Hz

frequency_max: 16kHz        # default 16 kHz

frequency_step: 10Hz        # default 10 Hz

The ranges are set once for the platform, not per band, to keep it simple for now.

A 10 Hz step gives 1600 positions: fine at high frequencies, coarse in the bass (20, 30, 40 Hz). mode: box lets a value be typed.

# EQ Switch

|  | EQ On | EQ Off |
| --- | --- | --- |
| always_on: false (bands default) | active | flat |
| always_on: true (fixed filters default) | active | active |

How Off is done is chosen when the YAML is compiled: with no always_on: true entries, the DSP bypass flag (one write, no click); otherwise the always_on: false biquads are written as all pass.

A gain or frequency changed while EQ is Off is kept and takes effect when EQ is turned On.

The switch is required when any entry has always_on: false. Otherwise a warning says it is not used.

# Biquads on Each Channel

An entry with channel: both uses one biquad on each channel; its slider writes both.

Cuts first: high pass, then low pass / notch / band pass, then shelves / equalizer / peaking. YAML order is kept within a group: fixed filters, then gain bands, then frequency bands.

Fixed filters + gain bands + frequency bands must be 15 or fewer per channel, otherwise a config error.

An entry with channel: both can sit at a different biquad on each channel.

# Examples

## Stereo

Both speakers get the same EQ from one set of sliders.

number:

- platform: tas58xx

gain_bands:                 # channel: both is the default

- name: 25Hz

frequency: 25Hz

# ... ISO 2/3-octave centres ...

- name: 16kHz

frequency: 16kHz

## Bi-amp (woofer left, tweeter right)

The crossover is made of fixed filters, two Butterworth biquads per side (Linkwitz-Riley 24 dB/octave). With always_on: true (the default) it stays in place when EQ is Off, so the tweeter never gets full range (v1 needed a warning for this).

audio_dac:

- platform: tas58xx

fixed_filters:

- {eq_filter: low_pass, frequency: 2kHz, channel: left}

- {eq_filter: low_pass, frequency: 2kHz, channel: left}

- {eq_filter: high_pass, frequency: 2kHz, channel: right}

- {eq_filter: high_pass, frequency: 2kHz, channel: right}

number:

- platform: tas58xx

gain_bands:                 # tone for the whole speaker

- name: Bass

eq_filter: low_shelf

frequency: 100Hz

- name: Treble

eq_filter: high_shelf

frequency: 8kHz

- name: Woofer 300Hz

frequency: 300Hz

channel: left

- name: Tweeter 5kHz

frequency: 5kHz

channel: right

| Biquad | Left (woofer) | Right (tweeter) |
| --- | --- | --- |
| 1 | LP 2 kHz | HP 2 kHz |
| 2 | LP 2 kHz | HP 2 kHz |
| 3 | Bass | Bass |
| 4 | Treble | Treble |
| 5 | Woofer 300Hz | Tweeter 5kHz |

## Adjustable subwoofer cut-off

number:

- platform: tas58xx

frequency_bands:

- name: Sub cut-off

eq_filter: low_pass

always_on: true         # stays in place when EQ is Off

# PBTL

Only the left channel is output, so channel: both applies to the left. channel: right is a config error.

# Breaking Changes (from the fork)

eq_mode moves from select: to switch: and only has On and Off.

Number EQ bands become gain_bands with a channel option.

left_fixed_filters / right_fixed_filters become fixed_filters with a channel option.

EQ presets are removed. A crossover preset can be built from fixed filters.

Fixed filters now stay active when EQ is Off unless they have always_on: false.

The saved EQ mode is reset once after updating.

# YAML Format Notes

Band and fixed filter lists accept either YAML form; both are valid and can be mixed:

gain_bands:

- name: Left 100Hz

frequency: 100Hz

channel: left

- {name: Left Bass, eq_filter: low_shelf, frequency: 80Hz, channel: left}

Commas without { } are invalid.

# Where the Filter Maths Lives

frequency_bands can change the frequency of any filter at runtime, so every filter's coefficient formula and the conversion to the DSP's 5.27 fixed point format must run on the device. Only fixed_filters could be calculated in Python at compile time. EQ is ESP32 only.

| Option | How | Assessment |
| --- | --- | --- |
| 1. Espressif ESP-DSP | ESP Component Registry, dsps_biquad_gen_* | Standard cookbook formulas only; not TI PurePath Console's conventions (notch / band pass by bandwidth) or the 5.27 conversion. Saves little code. Desk check against PurePath reference coefficients to confirm. |
| 2. Own library | Published on the ESP Component Registry (like esp-audio-libs), pulled in with add_idf_component | Keeps the helpers out of core, reusable by others, tested separately. Open: will maintainers accept a dependency not owned by ESPHome, and the publishing and maintenance it needs. Investigate. |
| 3. External component | Core tas58xx exposes a DSP write API; an external component adds the EQ | Technically possible, but core reviewers usually reject hooks with no in-tree user, and it gives little over the fork. |
| 4. All in core | Helpers in esphome/components/tas58xx | Simplest, but likely too much code for core review now that every filter runs on the device. |

Recommendation: investigate option 2 first, with the option 1 desk check alongside (ESP-DSP could be used inside the library if it matches). Option 3 is not pursued. Option 4 is the fallback.

# Open Points

Default of channel (both for now, may change).

Defaults of gain_min and gain_max.

Per-band slider ranges, if the platform-level ranges are not enough.

An initial value option for sliders.

An optional bandwidth for notch and band_pass (as an alternative to q_factor), to copy filters from TI PurePath Console.